TALLY BRIGHT User Guide Firmware v3.0 • 2-LED Edition

# 1.  Overview

Tally Bright is a compact WiFi-connected tally light designed for video production environments. It monitors your video mixer in real time and signals the camera's current status — live on air, in preview, or idle — through its two-colour LED indicator.

When connected to a compatible Osee GoStream mixer, the device updates automatically as you switch between camera sources. The device also provides a local web interface that lets you configure, monitor, and manually control it from any device on the same network.

# 2.  LED Indicators

The tally light communicates its status entirely through two LEDs. The table below describes every possible state.

| Tally State | LED Colour | LED Pattern |
| --- | --- | --- |
| LIVE (Program) | ● | Red — solid |
| PREVIEW | ● | Green — solid |
| Connecting to mixer | ● | Both LEDs on |
| No mixer host configured | ● | Both LEDs on (flashing) |
| Starting up / initialising | ● | Both LEDs on briefly |
| OFF (idle) | ● | Both LEDs off |

TIP: When the device first powers on, both LEDs flash briefly while the device initialises. A sequence of Red → Green → both LEDs on → off confirms a successful connection to the mixer.

# 3.  First-Time Setup

On first power-on, or after a configuration reset, Tally Bright creates its own temporary WiFi access point named Tally Bright Setup. Follow these steps to connect it to your network and mixer.

## 3.1  Connect to the setup network

- Power on the device. Both LEDs will illuminate.

- On your phone or laptop, open your WiFi settings and connect to the network named Tally Bright Setup.

- A captive portal page will open automatically. If it does not, open a browser and navigate to 192.168.4.1.

## 3.2  Enter your network credentials

- Select your WiFi network from the list and enter the password.

- Fill in the Mixer Configuration fields (see Section 4).

- Tap Save. The device will reboot and join your network.

NOTE: Leave the Mixer host field blank if you want the device to find the mixer automatically on the local network. Manual entry is also supported.

# 4.  Configuration Reference

All configuration settings can be entered through the setup portal during first-time setup, or updated later via the /config web endpoint (see Section 5).

| Setting | Description | Default |
| --- | --- | --- |
| Mixer host | IP address of the video mixer. Leave blank to use automatic network discovery. | auto discover |
| Mixer port | Network port used to communicate with the mixer. | 19010 |
| Device name | A friendly name for this tally unit, shown in API responses. | Camera 1 |
| Device ID | The input number on the mixer that this tally light represents. Use 1-based numbering (e.g., 1 = Camera 1). | 1 |
| Device mode | Operating mode. 0 = WiFi only (manual control). 1 = GoStream (automatic tally from mixer). See Section 6. | 1 |
| Brightness | LED brightness, from 10 (dim) to 255 (maximum). Note: on 2-LED hardware, the LEDs are either on or off; this value is stored for compatibility. | 50 |
| Protocol offset | Set to 1 for current GoStream firmware (1-based indexing). Set to 0 only if using an older mixer firmware that sends 0-based input numbers. | 1 |

# 5.  Web Interface

Once connected to your network, Tally Bright hosts a small web server. All commands are simple browser URLs — no app required. Replace <device-ip> with the IP address shown on your router, or use the device name if your network supports local name resolution.

## 5.1  Information endpoints

These endpoints are read-only and return status information in JSON format.

| URL | What it returns | Typical use |
| --- | --- | --- |
| http://<device-ip>/ | App name, version, and manufacturer details. | Confirm the device is reachable. |
| http://<device-ip>/version | App name and firmware version. | Check firmware version. |
| http://<device-ip>/status | Full live status: camera on air, preview active, WiFi connection, current program and preview indices. | Dashboard or integration polling. |
| http://<device-ip>/mixer | Configured mixer host and port. | Verify mixer connection settings. |
| http://<device-ip>/device | Device name and device ID. | Identify a specific unit. |

## 5.2  Configuration endpoint

The /config endpoint displays the full current configuration and allows any setting to be updated by appending parameters to the URL. Changes are saved immediately and persist across reboots.

http://<device-ip>/config

To update one or more settings, add them as URL parameters:

http://<device-ip>/config?device_name=Camera2&device_id=2

| Parameter | Accepted values | Example |
| --- | --- | --- |
| mixer_host | IP address string | ?mixer_host=192.168.1.50 |
| mixer_port | 1 – 65535 | ?mixer_port=19010 |
| device_name | Text, up to 40 characters | ?device_name=Camera+3 |
| device_id | 1 – 256 (1-based) | ?device_id=3 |
| device_mode | 0 (WiFi only) or 1 (GoStream) | ?device_mode=1 |
| device_brightness | 10 – 255 | ?device_brightness=128 |
| protocol_offset | 0 (old firmware) or 1 (current) | ?protocol_offset=1 |

## 5.3  Manual tally control

Use these endpoints to manually set the tally state without a mixer connection. This is useful when operating in WiFi-only mode (device_mode=0), for testing, or for integrating with custom automation systems.

| URL | Action | Optional parameter |
| --- | --- | --- |
| http://<device-ip>/program | Toggle program (LIVE) state on or off. | ?state=on or ?state=off |
| http://<device-ip>/preview | Toggle preview state on or off. | ?state=on or ?state=off |

When called without a ?state parameter, each endpoint toggles the current state. Examples:

http://<device-ip>/program?state=on      — forces LIVE (red LED)

http://<device-ip>/preview?state=on     — forces PREVIEW (green LED)

http://<device-ip>/program?state=off    — clears LIVE state

http://<device-ip>/preview              — toggles preview on or off

NOTE: Program mode always takes priority over preview mode. Setting both to 'on' at the same time will display red (LIVE).

## 5.4  System management

| URL | Action | Notes |
| --- | --- | --- |
| http://<device-ip>/boot | Immediately reboots the device. | Useful after changing settings that require a restart. |
| http://<device-ip>/reset?confirm=YES | Erases the WiFi configuration and restarts the setup portal. | The ?confirm=YES parameter is required. Omitting it returns an error without making any changes. |

WARNING: After a reset, the device will reappear as the Tally Bright Setup access point. You will need to repeat the first-time setup process.

# 6.  Operating Modes

## Mode 0 — WiFi Only

In this mode the device connects to your WiFi network and serves the web interface, but does not attempt to contact any mixer. All tally control is manual — use the /program and /preview endpoints to update the LED state from your own system, a streaming controller, or a browser shortcut.

This mode is suited to situations where:

- You do not have a GoStream mixer.

- You want to control tally from a custom automation script or broadcast controller.

- You are testing the device before integrating it into a production.

## Mode 1 — GoStream (Automatic)

This is the standard production mode. The device connects to the GoStream mixer over your local network and receives live program and preview updates. The LEDs update in real time whenever you switch sources on the mixer — no manual intervention needed.

The device supports both current and older GoStream mixer firmware. Use the Protocol offset setting (see Section 4) to match your mixer:

- Protocol offset 1 — use with current GoStream firmware.

- Protocol offset 0 — use only with older mixer firmware if the tally shows an incorrect camera number.

NOTE: If the mixer host is left blank, the device will attempt to find the mixer automatically on the local network. This requires the mixer to be powered on and reachable before the tally device boots.

# 7.  Automatic Mixer Discovery

When no mixer IP address is configured, Tally Bright will periodically search for a compatible mixer on the local network. Discovery runs every 30 seconds until a mixer is found.

For reliable automatic discovery:

- The mixer and all tally devices must be on the same network subnet.

- The mixer must be powered on and fully booted before the tally device searches.

- If discovery fails, enter the mixer IP address manually via the /config endpoint or the setup portal.

# 8.  Troubleshooting

| Symptom | Likely cause | Solution |
| --- | --- | --- |
| Both LEDs stay on after boot | Device cannot connect to the mixer. | Check that the mixer IP and port are correct. Verify the mixer is powered on and on the same network. Try entering the mixer IP manually via /config. |
| Both LEDs on (flashing) with no tally state | No mixer host is configured and auto-discovery has not found one. | Set the mixer host manually: http://<device-ip>/config?mixer_host=<mixer-ip> |
| LEDs show correct colour but wrong camera | Protocol offset mismatch between device and mixer firmware. | Try changing protocol_offset. If currently set to 1, try 0, or vice versa. |
| Device does not appear on the network | WiFi credentials incorrect, or device needs reset. | Hold the reset and repeat the first-time setup (Section 3). Ensure you are connecting to the same WiFi network as your mixer. |
| Setup portal does not open automatically | Captive portal blocked by device. | Open a browser and manually navigate to 192.168.4.1 while connected to the Tally Bright Setup network. |
| /reset returns an error | The ?confirm=YES parameter was missing or incorrect. | Use the exact URL: http://<device-ip>/reset?confirm=YES — the word YES must be uppercase. |
| Tally does not respond after mixer source change | TCP connection to mixer was lost. | The device will attempt to reconnect automatically. Wait a few seconds. If unresolved, use /boot to reboot the device. |

# 9.  Technical Specifications

| Item | Details |
| --- | --- |
| Firmware version | 3.0 |
| Network interface | WiFi (2.4 GHz) |
| Configuration interface | Captive portal web UI / HTTP REST API |
| Mixer protocol | Osee GoStream (TCP, port 19010 default) |
| Compatible mixers | Osee GoStream series |
| LED indicators | 2 × GPIO-driven LEDs (Red, Green) |
| Web server port | 80 (HTTP) |
| API response format | JSON |
| Manufacturer | makerUSA — makerusa.net |

# 10.  Legal & Support

For support, replacement parts, and accessories, visit makerusa.net.

© makerUSA. All rights reserved. Specifications subject to change without notice. Osee and GoStream are trademarks of their respective owners.